FUZHOU TOTINFO TECH., CO.,LTD                                         V2.1_20200617

D2 Commercial Desktop Barcode Scanner 
	Acquisition Performance

	 Sensor
	Image Type，CMOS Sensor

	Speed
	1/120 fps

	FOV
	Level 45°，Vertical 35°             

	Reading Performance

	 Precision
	≥ 4mil

	 Reading Mode
	Close to Read、Moving Reading

	 Depth of Field
 （1D）
	Testing Code 
	DOF(mm)
	Mobility (m/s)

	
	
	
	Level
	Vertical

	
	Code 128 10mil 18bytes Paper Code
	20~170
	2.5
	4

	
	EAN13 15mil 13bytes Paper Code
	10~280mm
	2
	6

	 Depth of Field
（2D）
	QR 10mil 160bytes Paper Code
	10~100mm
	2

	
	QR 20mil 11bytes Paper Code
	10~240mm
	3

	
	DM 15mil 100bytes Paper Code
	20~160mm
	1.5

	 Scan Angle
	Roll 360°，Pitch ±65°，Skew ± 60°

	Symbology
	2D Codes:

QR Code, Data Matrix, PDF417，Hanxin Code，Dotcode，OCR等 

	
	1D Codes: 

UPC-A, UPC-E, EAN-8, EAN-13, ISBN, Code 128, GS1 128, 

ISBT 128, Code 39, Code93, Code 11, Interleaved 2 of 5, 

Industrial 2 of 5,Matrix 25, Standard 25, Codabar, 

MSI/MSI PLESSEY, GS1 DataBar etc

	Print Contrast
	20% minimum reflectance

	LED

	Illumination
	RED LED

	Physical parameters

	Size(mm)
	85mm*85mm*155mm（L×W×H）

	Weight
	250g

	Interface

	Socket
	10P10C RJ45 Network Socket

	Interface 
	USB，RS232

	Scanning Mode
	Command Mode/ Sense Mode/ Trigger mode

	Notification
	Beeper、Green LED

	Cable（Optional）
	USB 2meter straight line、RS232 2meter straight line

	Environmental Parameter

	Operating Temperature
	-30℃ ~ 70℃ 

	Storage Temperature
	-40℃ ~ 80℃ 

	Humidity
	RH 5% ~ 95% (non-condensing)）

	Ambient Light
	Max.100,000 Lux 

	Electrical Parameter

	Input Voltage
	5V

	Operating Current
	＜500mA


Size （Tolerance+0.2mm）

[image: image1.png]78,0 ] o
>

198

1548

] ]

fe——850——+




[image: image2.png]



Exterior Photo
[image: image3.png]



- 3 -

